Boxwrx Speaker Data Menu (tips for entering Thiele-Small parameters)

The speaker data menu is interactive and can be confusing. Im most cases, you will enter Ts parameters directly from the manufacturers specifications. The following tips will help you avoid mistakes

S(d) -sq cm -> Entry required. Enter a value in either box. The other will be calculated automatically

Piston diameter – mm -> be calculated automatically

Free air resonance – Hz -> Entry required

(Bl)-squared/R -> ignore this entry. Boxwrx will calculate it.

Millihenries/R -> Entry required. Divide the specified inductance by R(e) before entering. If you don’t know, enter 0.25

Min. Impedance / R -> Entry required. Divide the specified value by R(e) before entering it. If you don’t know, enter 1.10

Q of mechanical system ->   entry required

Max input – watts -> optional entry. Used only to graph maximum output

Peak excursion – mm -> optional entry. Used only to graph maximum output

Q(ts) -> entry required

V(as) – liters -> Entry required

Check all your entries before proceeding. Some may have changed, depending on the sequence in which they were entered. You can save time by entering Q(ts) last.

Plans for after spring break – start cutting immediately